# Comprehensive Analysis ofi Electric Vehicle Charging Infirastructure: OCPP Protocol Evolution and Smart Charging.

The transition toward a decarbonized transportation sector is fiundamentally reliant on the seamless integration ofi electric vehicles (EVs) into the existing power grid. At the center ofi this integration lies the communication protocols that govern the exchange ofi infiormation between hardware, software, and the energy network. The Open Charge Point Protocol (OCPP) has emerged as the global standard fior managing these complex interactions, evolving firom a basic connectivity tool into a sophisticated orchestration firamework.1 This report provides an exhaustive technical examination ofi OCPP versions 1.6 and 2.0.1, the mechanisms ofi smart charging, and the strategic considerations governing infirastructure deployment in a rapidly maturing global market.

# The Genesis and Strategic Role ofi OCPP

The Open Charge Point Protocol is an open-source, vendor-neutral communication standard developed and maintained by the Open Charge Alliance (OCA), a global consortium ofi EV charging hardware manufiacturers, software developers, and grid operators.2 Befiore the widespread adoption ofi OCPP, the industry was firagmented by proprietary protocols that tethered charging station operators to specific hardware-software combinations, creating significant risks ofi "stranded assets" ifi a manufiacturer went out ofi business or ifi their software fiailed to meet evolving network needs.2

OCPP was designed to eliminate these barriers by providing a unifiorm method ofi communication between the Charging Station (refierred to in earlier versions as the Charge Point or CP) and the Central Management System (refierred to as the Charging Station Management System or CSMS).1 By standardizing the language ofi charging, OCPP enables a "plug-and-play" ecosystem where any certified CSMS can manage any certified charging station, regardless ofi the vendor.2 This interoperability is the linchpin ofi a scalable, competitive market, allowing operators to select best-ofi-breed components fior their specific use cases while ensuring long-term flexibility.1

The fiundamental role ofi OCPP is to manage the lifiecycle ofi a charging session, which involves several critical fiunctions:

- Initialization and Provisioning: Ensuring the station can identifiy itselfi and receive its initial configuration upon startup.7

- Authorization: Validating the identity ofi a user via RFID, mobile app, or vehicle-based certificates to prevent unauthorized use.1

- Transaction Management: Recording the start and end times ofi charging sessions and

the total energy consumed fior billing and telemetry purposes.6

- Remote Control and Availability: Allowing operators to start, stop, or reset chargers remotely and report their real-time availability to navigation apps.1

- Diagnostics and Maintenance: Reporting errors and operational metrics to enable remote troubleshooting and minimize downtime.7

# Technical Deconstruction ofi OCPP 1.6

Released in 2015, OCPP 1.6 marked the protocol's maturity and remains the most widely implemented version in the global charging ecosystem.3 Its adoption was driven by the introduction ofi the "JSON over WebSockets" implementation (OCPP 1.6J), which addressed the significant limitations ofi the original SOAP-based versions.8

## Architecture and Communication Models

OCPP 1.6 is architecturally "connector-centric," meaning the protocol logic treats each physical charging outlet as a largely independent endpoint.9 This flat structure was efficient fior early charging hardware, which typically consisted ofi single or dual-socket pedestals with limited internal processing power.

The protocol supports two primary communication bindings:

- SOAP/XML: A legacy protocol using the Simple Object Access Protocol over HTTP. In this model, the charging station acts as a client that must periodically poll the CSMS fior commands, which can introduce latency and high overhead due to the verbose nature ofi XML.8

- JSON over WebSockets (1.6J): The modern standard fior 1.6 deployments. By using WebSockets, the station and the CSMS maintain a persistent, bi-directional connection.8 This allows the CSMS to push messages—such as a remote start command—to the charger instantly, significantly improving the user experience and enabling real-time grid interaction.10

## Core Functional Profiles and Features

OCPP 1.6 is organized into fiunctional profiles, which allow implementers to select the specific capabilities required fior their business case.1

  - Core Profile: Includes essential messages such as BootNotification, Authorize, StartTransaction, and StopTransaction. This profile ensures the basic ability to initiate and record a charging session.8

  - Firmware Management Profile: Facilitates remote updates via UpdateFirmware and diagnostic collection via GetDiagnostics. This is critical fior maintaining security and operational health across a distributed network.1

  - Local Auth List Management: Allows the CSMS to send a list ofi authorized IDs

(SendLocalList) to the charger's local cache. This ensures that users can still charge ifi the station loses its network connection.1

  - Reservation Profile: Enables a user to "lock" a connector fior a set period using the ReserveNow message, preventing other drivers firom using it until the reservation expires or the user arrives.1

  - Smart Charging Profile: The introduction ofi smart charging in 1.6 was a major milestone. It allows the CSMS to send a SetChargingProfile, which defines a series ofi time-stamped power limits. These profiles can be based on specific transactions (TxProfile),

connector-wide defiaults (TxDefiaultProfile), or station-wide maximums (ChargingStationMaxProfile).3

## Security and Technical Constraints

Security in OCPP 1.6 is primarily fiocused on transport-layer encryption using Transport Layer Security (TLS).9 While TLS protects the data in transit firom eavesdropping, the 1.6 specification lacks robust requirements fior mutual authentication and certificate management.

Authorization is often handled through simple username-password tokens, which are

vulnerable to credential harvesting and replay attacks ifi the underlying network is compromised.9

The limitations ofi the 1.6 firamework became apparent as charging stations became more complex. Its flat architecture cannot easily model modern "split" units (where a central power cabinet fieeds multiple remote dispensers) or multi-standard fiast chargers that share a single power module.9 Furthermore, its diagnostics are limited to high-level status notifications; ifi a station reports a "Faulted" status, the operator often has no way to know ifi the fiailure occurred in the RFID reader, the contactor, or the network modem without an on-site visit.11

# The Strategic Shift: Deep Dive into OCPP 2.0.1

OCPP 2.0.1, released in 2020, was not an incremental update but a comprehensive redesign intended to transfiorm the charging station into a sophisticated node within the smart grid.2 It addresses the fioundational flaws ofi 1.6 while introducing native support fior next-generation fieatures like Plug & Charge and Vehicle-to-Grid (V2G).11

## The Hierarchical Device Model

The most profiound change in 2.0.1 is the transition to a three-tier hierarchical device model:

Charging Station > EVSE > Connector.7

  - Charging Station: The physical enclosure containing logical controllers, displays, and internal sensors.7

  - EVSE (Electric Vehicle Supply Equipment): A logical unit capable ofi delivering power to one vehicle at a time. A station may contain multiple EVSEs, each managing its own power conversion hardware.9

  - Connector: The physical plug or socket. An EVSE might have multiple connectors (e.g., one CCS and one Type 2), but the hierarchy recognizes that only one can be active at a time fior that specific EVSE.10

This modular structure allows the CSMS to see and control every component ofi the station. Variables can be assigned to any component (e.g., a fian speed variable on the cooling component) and monitored in real-time.11 This granularity enables "inventory reporting," where the station can tell the CSMS exactly what hardware is inside it, drastically simplifiying maintenance and firmware management.7

## Enhanced Security Architecture

While 1.6 provided basic encryption, 2.0.1 implements a robust "Zero Trust" security firamework through three standardized security profiles 7:

- Security Profile 1: Unencrypted (legacy/testing only).

- Security Profile 2: TLS with basic authentication (username/password).7

- Security Profile 3: Mutual TLS (mTLS) with X.509 certificates. In this profile, both the charging station and the CSMS must prove their identity via digital certificates, making it nearly impossible fior unauthorized devices to spoofi a station or a management server.7

Beyond transport security, 2.0.1 adds "Security Events," where the station can alert the CSMS ofi physical tampering, such as the door being opened or repeated fiailed authorization attempts.7

## Transaction and Monitoring Improvements

Transaction handling in 2.0.1 has been unified into a single TransactionEvent message.14 In 1.6, a session required separate StartTransaction, StopTransaction, and MeterValues messages, which could easily get out ofi sync ifi the connection was unstable.9 In 2.0.1, every event is a TransactionEvent with a sequence number, allowing the CSMS to perfiectly reconstruct the session history even ifi the messages arrive out ofi order.8

Monitoring is also transfiormed through the SetMonitoringBase and GetMonitoringReport commands.7 Operators can now set custom alerts: fior example, the station can be configured to send an alert only ifi its internal temperature exceeds 70°C, rather than bombarding the CSMS with constant status updates. This "exception-based reporting" reduces data costs and fiocuses operator attention on actual issues.7

## Advanced Charging Management and ISO 15118

A critical pillar ofi 2.0.1 is its native compatibility with ISO 15118, which defines the communication between the vehicle and the charger.10 This enables several key fiunctionalities:

- Plug & Charge: The vehicle automatically negotiates authorization and payment with the charger, allowing the driver to simply plug in and walk away.1

- State ofi Charge (SOC) Reporting: The station can relay the vehicle's actual battery level

and requested energy amount to the CSMS, allowing fior much more accurate smart charging profiles.14

- Grid Integration: The station acts as a transparent bridge fior complex power schedules, supporting fiuture V2G and bidirectional charging use cases.19

# Technical Comparison Table: OCPP 1.6 vs. OCPP 2.0.1

The fiollowing table provides a high-level technical contrast ofi the two protocols across essential operational metrics.

| Feature | OCPP 1.6 | OCPP 2.0.1 |
| --- | --- | --- |
| Architectural Model | Flat, Connector-Centric 10 | Hierarchical, Object-Oriented Device Model 9 |
| Communication Binding | SOAP/XML and JSON/WebSockets (1.6J) 3 | JSON over WebSockets only 8 |
| Security | Basic TLS; single-sided auth 9 | Mandatory TLS 1.2+; Mutual TLS (mTLS) support 9 |
| Device Management | Limited to basic configuration keys 9 | Comprehensive modular monitoring & configuration 16 |
| Transaction Logic | Multiple messages (Start, Stop, MeterValue) 9 | Unified TransactionEvent with sequence numbering 8 |
| Smart Charging | Static profiles; limited fieedback firom vehicle 3 | Dynamic profiles; integrated ISO 15118 SOC data 10 |
| Message Structure | Fragmented; requires chronological order 9 | Streamlined; robust against out-ofi-order delivery 8 |
| ISO 15118 Support | No native support (requires proprietary extensions) 11 | Native support fior Plug & Charge and V2G 10 |
| Bandwidth Efficiency | JSON (1.6J) is efficient but verbose 10 | Supports WebSocket Compression fior high |

|  |  | traffic 9 |
| --- | --- | --- |
| Future Scalability | Low; limited by flat architecture 9 | High; designed fior MW charging and V2X 8 |

# Why OCPP 1.6 Continues to Dominate the Industry

Despite the technical superiority ofi OCPP 2.0.1, an overwhelming majority ofi current deployments—estimated at over 90% in most regions—remain on OCPP 1.6.5 This persistence is driven by a combination ofi economic, technical, and ecosystem fiactors.

## Market Maturity and Stability

OCPP 1.6 is considered a "mature" standard.10 Having been in use since 2015, its edge cases are well-understood, and a massive ecosystem ofi tested hardware and software already exists.11 For most standard use cases, such as workplace charging or single-socket home chargers, the fieatures ofi 1.6 are entirely sufficient.13

## Hardware and Implementation Complexity

The move to 2.0.1 is not just a software patch; it requires significantly more powerfiul hardware.13 The requirements fior mTLS encryption and the management ofi a complex hierarchical device model demand more memory and processing speed than many legacy charger controllers can provide.11 Manufiacturers fiace a steep learning curve and higher development costs to implement the 2.0.1 stack, which can reach over 100 discrete message types compared to the 30-40 fiound in most 1.6 implementations.8

## Cost and Deployment Scale

Upgrading a massive network firom 1.6 to 2.0.1 is capital-intensive.8 Since the two protocols are not backward compatible, an operator must either replace hardware or maintain two parallel management systems, increasing operational overhead.9 For large-scale CPOs (Charge Point Operators) with thousands ofi existing units, the business case fior migration only becomes compelling when specific 2.0.1-only fieatures, such as Plug & Charge or fiederal subsidies, become essential.23

## Case Study: Zest and the Pragmatic Use ofi OCPP 1.6

A primary example ofi this pragmatic approach is Zest, a Leeds-based Charge Point Operator (CPO) and one ofi the fiastest-growing networks in the UK. Zest’s strategy provides a real-world illustration ofi why 1.6 remains the baseline fior reliable infirastructure:

- Proven Reliability: Zest maintains a 99% national uptime rate by utilizing the stable 1.6 standard, which handles core fiunctions like remote start/stop and real-time monitoring effectively without the complexity-induced instability ofi newer versions.

- Hardware Agnosticism: As an operator that must work with various hardware manufiacturers, Zest relies on 1.6 because it is supported by virtually all major vendors. This allows them to select the best physical charger fior a location (e.g., dual-socket fiast or rapid chargers) while managing them through a single, standard platform.

- Strategic Growth: Backed by the UK government's Charging Infirastructure Investment Fund (CIIF) and certified as a B Corp, Zest prioritizes accessible, reliable charging today. They demonstrate that while 2.0.1 is the "fiuture," 1.6 is the most effective tool fior achieving national scale and consistent perfiormance in the present market.

# Smart Charging: Definition and Grid Necessity

Smart charging is the intelligent management ofi an EV charging session where the power draw is adjusted based on data-driven signals firom the vehicle, the grid, or the site owner.1 It is the alternative to "dumb" or unmanaged charging, where a vehicle draws its maximum possible power as soon as it is plugged in, regardless ofi the impact on the electrical system.20

As EV adoption grows, smart charging becomes a physical necessity fior the grid. A study in Germany fiound that ifi local EV penetration reaches 25% without smart charging, peak loads on the distribution grid could increase by 30%, potentially causing blackouts or necessitating multi-billion dollar infirastructure upgrades.19 Smart charging can reduce this peak load impact by over 50%, allowing fior higher EV adoption using existing wires.19

# Technical Mechanisms ofi Smart Charging

Smart charging operates through several fiundamental technical layers:

## Load Balancing

Load balancing ensures that the total power demand ofi multiple chargers stays within the capacity ofi the local electrical supply.26

- Mechanism: The system monitors the available current and dynamically sends commands to each charger to increase or decrease their output based on the number ofi vehicles connected.28

- Dynamic Power Allocation: This is an advanced fiorm ofi load balancing that considers the building's other loads (e.g., elevators, AC units). Ifi the building's energy use spikes, the EV chargers are throttled down in real-time to avoid tripping the main fiuse.26

## Grid Interaction and Scheduling

Charging stations can interact with the wider grid through two primary methods:

- Direct Load Control (DLC): The utility or a third-party aggregator directly sends signals to the CSMS to reduce load during a grid emergency.25

- Price-Based Response: The CSMS receives "Time-ofi-Use" (TOU) tariff data and

automatically schedules charging fior hours when electricity is cheapest and most abundant (e.g., 2 AM).19

# Categories and Types ofi Smart Charging

Smart charging is often categorized by its complexity and the direction ofi energy flow.

| Type | Technical Description | Real-World Example |
| --- | --- | --- |
| V1G (Unidirectional) | The charging rate can be modulated up or down, but power only flows firom the grid to the car.19 | Managing a workplace fleet to charge slowly over 8 hours to avoid peak demand charges.19 |
| Static Load Balancing | A fixed amount ofi power is shared equally among active sockets.26 | A parking garage with a 100 kW limit; ifi 10 cars plug in, each gets 10 kW.26 |
| Dynamic Load Balancing | Power allocation shifts based on building-wide real-time demand.26 | A home charger that slows down when the electric oven and HVAC are turned on.26 |
| Demand Response | Reducing charging load in response to utility "event" signals.24 | A utility paying a CPO to reduce network load by 20% fior two hours during a heatwave.30 |
| V2G (Bidirectional) | Power can flow firom the car battery back into the grid.19 | School buses fieeding power back into the grid during summer months in White Plains, NY.30 |
| V2H / V2B | Using the EV battery to power a home or building during an outage.20 | A Nissan Leafi powering a residential load to avoid high peak electricity prices.21 |

# Smart Charging Implementation within OCPP

OCPP provides the standardized structure needed to execute these complex energy management strategies.

## Smart Charging in OCPP 1.6

In 1.6, smart charging is implemented through "Charging Profiles." A profile is essentially a list ofi periods with an associated power or current limit.3

- TxProfile: Attached to a specific transaction ID. It ensures that a car's charging schedule persists even ifi other cars plug in or out.9

- TxDefiaultProfile: Applied automatically to any car that plugs in, unless a specific TxProfile is sent. This is usefiul fior "always-on" load management.9

- ChargingStationMaxProfile: Sets the overall limit fior the entire hardware unit.

The primary limitation in 1.6 is that the CSMS "guesses" what the car needs based on previous data, as the protocol doesn't natively report the vehicle's state ofi charge (SOC).11

## Improvements in OCPP 2.0.1

OCPP 2.0.1 significantly enhances smart charging by adding data-rich fieedback loops.10

- Energy Requirements: Through ISO 15118, the car can tell the charger exactly how many kWh it needs and when it plans to leave.14

- Charging Schedule Negotiation: The CSMS can send a proposed schedule to the vehicle, which can then accept or counter-propose based on the driver's needs.14

- External Constraints: Version 2.0.1 introduces a notification mechanism fior "External Limits," allowing the station to quickly notifiy the CSMS ifi a local Energy Management System (EMS) has imposed a power restriction.18

# The Impact and Benefits ofi Smart Charging

The implementation ofi smart charging delivers systemic benefits across the energy value chain.

- Grid Stability: By "filling the valleys" and "shaving the peaks," smart charging prevents transfiormer overloads and stabilizes grid firequency.19 V2G technology can even provide "spinning reserves," where car batteries act as a fiast-responding resource to balance sudden drops in renewable production.19

- Renewable Integration: EVs can be scheduled to charge specifically when solar or wind production is at its peak. This prevents renewable energy firom being "curtailed" (wasted) and ensures that the EV fleet is powered by the cleanest possible energy.20

- Cost Reduction: For the driver, smart charging enables off-peak charging that can reduce annual fiueling costs by up to $700.31 For the operator, it avoids expensive "demand charges"—penalties levied by utilities fior high momentary power draw.26

- Efficiency: Dynamic load balancing ensures that every available amp is used effectively. Ifi one car finishes charging early, its power is immediately redistributed to the cars that are still charging, maximizing the throughput ofi the station.27

# Real-World Implementation and Case Studies

The fiollowing examples demonstrate the practical application ofi smart charging and OCPP-based orchestration at scale.

  - UCLA Smart Grid Energy Research Center (SMERC): UCLA deployed a bidirectional charging system in Santa Monica that integrated smart charging with local battery storage.36 The project used unique algorithms to demonstrate peak shaving and load leveling, proving that managed charging can sustain grid resiliency even in high-demand municipal areas.36

  - The Virta V2G Platform: Virta operates several V2G hubs, including one at their headquarters, where Nissan vehicles provide power back to the building to balance the electricity demand ofi office lighting and AC systems.21 This implementation shows how bidirectional charging can transfiorm a fleet into a "virtual power plant" (VPP) that provides flexibility to the local distribution network.21

  - Con Edison V2G Buses: In White Plains, New York, five electric school buses fieed power back into the grid during times ofi high demand.30 Because buses have large batteries and consistent schedules, they are ideal candidates fior V2G services, generating revenue fior the school district while supporting the local utility during heatwaves.30

  - Shell and Last Mile Solutions Migration: One ofi Europe's largest migrations involved transfierring 100,000 chargers firom Shell's legacy systems to Last Mile Solutions (LMS).37 The success ofi this massive project relied entirely on the fiact that the hardware was OCPP-certified, allowing the new network to take control ofi the chargers remotely using standard ChangeConfiguration and SetNetworkProfile commands without any physical site visits.37

# Synthesis and Conclusion

The evolution firom OCPP 1.6 to 2.0.1 represents a fiundamental shift in the design philosophy ofi EV charging. While version 1.6 successfiully standardized basic operations and allowed the industry to scale, it was essentially a "dumb" connectivity tool. Version 2.0.1, with its hierarchical device model, zero-trust security, and native ISO 15118 support, turns the charging station into an intelligent, grid-aware asset.

The transition to 2.0.1 is being accelerated by government mandates like the US NEVI program and the EU AFIR regulation, which recognize that a secure, transparent, and manageable charging network is critical to national energy security.23 However, the continued persistence ofi

1.6 reminds us that stability and cost-effectiveness are equally vital in a mass-market environment.

Smart charging is the technology that bridges the gap between these protocols and the electrical grid. By modulating the charging ofi millions ofi EVs, we can turn a potential threat to grid stability into a powerfiul tool fior decarbonization. As V2G moves firom pilot projects to

mainstream commercial reality, the orchestration provided by OCPP will be the primary mechanism fior balancing the needs ofi the vehicle, the driver, and the planet. The fiuture ofi EV infirastructure is not just about the delivery ofi electricity; it is about the intelligent management ofi data and energy in a unified, open ecosystem.

Sources des citations

- The OCPP Handbook (2025) - AMPECO, consulté le mars 16, 2026, https://www.ampeco.com/guides/complete-ocpp-guide/

- Open charge point protocol, consulté le mars 16, 2026, https://openchargealliance.org/protocols/open-charge-point-protocol/

- OCPP 1.6 - Open Charge Alliance, consulté le mars 16, 2026, https://openchargealliance.org/protocols/ocpp-protocols/ocpp-1-6/

### OCPP 1.6 Test Procedure & Test Plans - Open Charge Alliance, consulté le mars 16, 2026,

https://openchargealliance.org/wp-content/uploads/2025/09/02.-Test-Procedure- Test-Plans_OCPP16_v2.4_final.pdfi

- October 24, 2025 Honorable Michelle L. Phillips State ofi New York Public Service Commission Building Three Empire State Plaza, consulté le mars 16, 2026, https://documents.dps.ny.gov/public/Common/ViewDoc.aspx?DocRefiId={909217 9A-0000-CA35-817A-923ABE4DFDE0}&DocTitle=Tesla%20Comments%20on%2 0Make-Ready%20Communication%20Standards

- Open charge point protocol (OCPP): Definition, versions, benefits, and software - Monta, consulté le mars 16, 2026, https://monta.com/en/blog/ocpp/

- A comprehensive guide to OCPP 2.0.1 - eDRV, consulté le mars 16, 2026, https://www.edrv.io/guide/ocpp-2-0-1-comprehensive-guide

### The Impact ofi OCPP in the EV Industry and Evolution firom OCPP1.6 to OCPP2.1 - Embedded, consulté le mars 16, 2026,

https://www.embedded.com/the-impact-ofi-ocpp-in-the-ev-industry-and-evolut  ion-firom-ocpp1-6-to-ocpp2-1/

### OCPP 1.6 vs. OCPP 2.0: A Comprehensive Comparison - Ampcontrol, consulté le mars 16, 2026,

https://www.ampcontrol.io/post/ocpp-1-6-vs-ocpp-2-0-a-comprehensive-comp arison

### OCPP 1.6 vs OCPP 2.0: A Detailed Comparison fior EV Chargers, consulté le mars 16, 2026,

https://www.luxmanenergy.com/ocpp-1-6-vs-ocpp-2-0-a-detailed-comparison-fi  or-ev-chargers/

### OCPP 1.6 vs OCPP 2.0.1: Key differences fior EV charging, CSMS, and charger firmware, consulté le mars 16, 2026,

https://chargelab.co/blog/ocpp-1.6-vs-ocpp-2.0.1-key-differences-fior-ev-chargin g-csms-and-charger-firmware

- Certification OCPP 1.6 - Open Charge Alliance, consulté le mars 16, 2026, https://openchargealliance.org/certificationocpp/certification-ocpp-1-6/

### OCPP 2.0.1 vs OCPP 1.6 — What's the Difference and What's ..., consulté le mars

16, 2026,

https://mikebolshakov.medium.com/ocpp-2-0-1-vs-ocpp-1-6-whats-the-differen  ce-and-what-s-coming-next-bb913d377863

- What is new in OCPP 2.0.1 - Open Charge Alliance, consulté le mars 16, 2026, https://openchargealliance.org/wp-content/uploads/2024/01/new_in_ocpp_201-v 10.pdfi

- OCPP 2.0.1 - Open Charge Alliance, consulté le mars 16, 2026, https://openchargealliance.org/protocols/ocpp-protocols/ocpp-2-0-1/

- Device Model: Addressing Components and Variables - OCPP Spec, consulté le mars 16, 2026, https://ocpp-spec.org/docs/ocpp_2_0/architecture/device-model/

### OCPP 2.0.1: The Future ofi EV Charging Protocols - eInfiochips, consulté le mars 16, 2026,

https://www.einfiochips.com/blog/ocpp-2-0-1-as-the-fiuture-ofi-ev-charging-prot ocols/

### OCPP 1.6J vs 2.0.1: A Technical Deep Dive & Upgrade Guide - LinkPower, consulté le mars 16, 2026,

https://www.elinkpower.com/news/ocpp-open-charge-point-protocol-firom-1-5-t  o-2-1-in-ev-charging/

- Powering the era ofi e-mobility with smart charging - Eurelectric, consulté le mars 16, 2026, https://www.eurelectric.org/in-detail/smartcharging/

### Electric-Vehicle Smart Charging – Innovation Landscape Briefi - IRENA, consulté le mars 16, 2026,

https://www.irena.org/-/media/Files/IRENA/Agency/Publication/2019/Sep/IRENA_E V_Smart_Charging_2019.pdfi

- Vehicle-to-Grid (V2G): Everything you need to know - Virta, consulté le mars 16, 2026, https://www.virta.global/vehicle-to-grid-v2g

### OCPP 1.6 vs 2.0 vs. 2.1 Comparing: Benefits, Limitations, and Adoption Trends,

consulté le mars 16, 2026,

https://lembergsolutions.com/blog/ocpp-16-vs-20-vs-21-comparing-benefits-lim  itations-and-adoption-trends

### OCPP Protocol Explained: Why Open Standards Matter fior EV ..., consulté le mars 16, 2026,

https://www.zest.uk.com/news/ocpp-protocol-explained-why-open-standards-m  atter-fior-ev-charging

- (PDF) Smart Charging Solutions fior Electric Vehicles: An AI-Driven Approach to Load Balancing and Grid Integration - ResearchGate, consulté le mars 16, 2026, https://www.researchgate.net/publication/398004553_Smart_Charging_Solutions

### _fior_Electric_Vehicles_An_AI-Driven_Approach_to_Load_Balancing_and_Grid_Int egration

- Electric Vehicle Managed Charging: Forward-Looking ... - Publications, consulté le mars 16, 2026, https://docs.nrel.gov/docs/fiy22osti/83404.pdfi

- Static vs. Dynamic Load Balancing: How to Choose? - BESEN, consulté le mars 16, 2026, https://www.besen-group.com/static-vs-dynamic-load-balancing/

### STATIC VS. DYNAMIC LOAD BALANCING IN EV CHARGING | Teltonika Energy,

consulté le mars 16, 2026,

https://www.teltonika-energy.com/newsroom/static-vs-dynamic-load-balancing-i n-ev-charging

### EV Charging Load Management: A Comprehensive Guide | Beny New Energy, consulté le mars 16, 2026,

https://www.beny.com/ev-charging-load-management-explained/

### Dynamic load balancing – essential in larger installations | ctek.com, consulté le mars 16, 2026,

https://www.ctek.com/se/nyheter/dynamic-load-balancing-essential-in-larger-ins tallations

- Bidirectional Charging and Electric Vehicles fior Mobile Storage | Department ofi Energy, consulté le mars 16, 2026, https://www.energy.gov/cmei/fiemp/bidirectional-charging-and-electric-vehicles- mobile-storage

- EV Grid Interaction - National Governors Association, consulté le mars 16, 2026, https://www.nga.org/wp-content/uploads/2020/12/EV-Grid-Interaction.pdfi

### Enhancing grid resilience: EV charging as a key strategy amid natural disasters, consulté le mars 16, 2026,

https://www.renewableenergyworld.com/electric-vehicle/vehicle-to-grid/enhanci ng-grid-resilience-ev-charging-as-a-key-strategy-amid-natural-disasters/

### Optimization ofi Load Balancing through Vehicle-to-Grid - Patsnap Eureka, consulté le mars 16, 2026,

https://eureka.patsnap.com/report-optimization-ofi-load-balancing-through-vehi cle-to-grid

- Critical Elements ofi Vehicle-to-Grid (V2G) Economics - Publications, consulté le mars 16, 2026, https://docs.nrel.gov/docs/fiy17osti/69017.pdfi

### Electric Vehicle-to-Grid (V2G) Technologies: Impact on the Power Grid and Battery - MDPI, consulté le mars 16, 2026,

https://www.mdpi.com/2071-1050/14/21/13856

- Final Project Report, Demonstrating Plug-in Electric Vehicles Smart Charging and Storage Supporting the Grid, consulté le mars 16, 2026, https://www.energy.ca.gov/sites/defiault/files/2021-06/CEC-500-2018-020.pdfi

- Migrating Charging Stations, consulté le mars 16, 2026, https://openchargealliance.org/ocpp-infio-whitepapers/migrating-charging-statio ns/

### 6 Best Practices For EV Charger Network Migration - EV Connect, consulté le mars 16, 2026,

https://www.evconnect.com/blog/best-practices-ev-charger-network-migration/

- Whitepapers Archive - Open Charge Alliance, consulté le mars 16, 2026, https://openchargealliance.org/ocpp-infio-whitepapers/